# Inspection Checklist

| Location: | 55 Rising Place, Kuraby | Date: | 21/11/25 | Time | 15:30 |
| --- | --- | --- | --- | --- | --- |
| Owner: | Doug Matthews | RP: |  |  |  |
| Tennant: |  | Telephone: |  |  |  |
| Email: |  | Referrer: | Direct |  |  |

Gates

Opens outwards (QLD Dev Code MP3.4 Sch 1 – 15)

Latches from any position (AS1926.1 2.5.3)

Stays latched when pulled (AS1926.1 2.5.4.1)

Closes after the application of a 25kg weight (AS1926.1 3.4)

Latch a minimum of 1500mm (AS1926.1 2.5.4.2) or 150 below the top of the gate (AS1926.1 Fig 2.6(a)) or 450mm shielding fitted (AS1926.1 Fig 2.6(b)/(c))

Gap between gate and fence less than 10mm (AS1926.1 2.5.4.1)

Less than 100mm above ground level (AS1926.1 2.4)

At least 1200mm high (AS1926.1 2.1)

Hinges exceeding 10mm in diameter and less than 900mm apart have non-climbable cap fitted (AS1926.1 2.3.4)

No items to prop open

Pool Barrier - Fences

The height is at least 1200mm (AS1926.1 2.1)

Less than 100mm above the finished ground level (AS1926.1 2.4)

Garden beds / Rising ground within NCZ (QLD Dev Code MP3.4 Fig 3)

Gaps in vertical bars less than 100mm AS1926.1 Fig 2.2)

Horizontal members more than 900mm apart or (AS1926.1 2.3.4 / QDC MP3.4 Schedule 1)

Gap between palings less than 10mm and horizontal members on the inside of the fence (AS1926.1 2.3.6)

Screws, nails and fixings secure (AS1926.1 2.1)

All panels in place (AS1926.1 2.1)

Pool Barrier - Boundary Fences

1800mm high or Non-Climbable Zone is on the outside of the fence (AS1926.1 2.3.1 / QDC MP3.4 Schedule 1)

At least 900mm between horizontal members (AS1926.1 2.3.4 / QDC MP3.4 Schedule 1)

All panels in place (AS1926.1 2.1)

Non-Climbable Zone

Fences less than 1800mm high - arc of 900mm clear from the top of the fence (QDC MP3.4 Fig 1)

1200mm in the bottom quadrant –(rising ground,retaining walls, fence rails) (QDC MP3.4 Fig 1)

Protrusions and indentations less than 10mm (AS1926.1 2.3.3)

No climbable trees lower than 900mm and no overhanging climbable objects within the upper quadrant (QDC MP3.4 Fig 1)

Pot plants (QDC MP3.4 Fig 6)

Plastic or lightweight furniture inside the 300mm ACA (QDC MP3.4 Fig 1-9)

Strength and rigidity

Squeeze test (AS1926.1 Appendix A)

Pushover test (AS1926.1 Appendix B)

Rust, rot, loose or missing fixings (AS1926.1 2.1)

Windows

Do not open more than 100mm or have security barriers fitted (AS1926.1 2.7)

Doors

Not acceptable for outdoor pools (QDC MP3.4 Schedule 1/ AS1926.1 2.8)

Fence between all buildings and pool (QDC MP3.4 Figure 32)

Resuscitation Sign

300 x 300 minimum (Building Regulation 2006 Part 4 Division 1 13A (2)(b))

Durable and weatherproof (Building Regulation 2006 Part 4 Division 1 13A (2)(c))

Visible from within pool area (Building Regulation 2006 Part 4 Division 1 13A (2)(a))

2 breaths / 30 compressions (Building Regulation 2006 Part 4 Division 1 13A (2)(f))

DRABCD / not check for pulse (Building Regulation 2006 Part 4 Division 1 13A (2)(f))

Bonding

Fencing within arms reach (1250mm) of waters edge is bonded AS/NZS3000)

Chemicals

Stored and out of reach of children

PPE available for when using chemicals

Notes:

Inspectors Name: _________________________